RALPH MAGO

CPE301 – SPRING 2016

# Design Assignment 2

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST | Y |  |
| 1. | INITIAL CODE OF TASK 1/A | Y |  |
| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 1/B | Y |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 1/C | Y |  |
| 4. | INITIAL CODE OF TASK 2/A | Y |  |
| 5. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B | Y |  |
| 6. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/C | Y |  |
| 7. | SCHEMATICS | Y |  |
| 8. | SCREENSHOTS OF EACH TASK OUTPUT | Y |  |
| 9. | SCREENSHOT OF EACH DEMO | Y |  |
| 10. | VIDEO LINKS OF EACH DEMO | Y |  |
| 11. | GITHUB LINK OF THE DA | Y |  |

| 0. | COMPONENTS LIST |  |  |
| --- | --- | --- | --- |

| ATMega328P-PU | 1 |
| --- | --- |
| 330 Ω Resistor | 11 |
| 10-LED Bar | 1 |
| Green LED | 1 |
| Breadboard | 1 |

| 1. | INITIAL CODE OF TASK 1/A |  |  |
| --- | --- | --- | --- |

;

; DA2T1.asm

;

; Created: 3/8/2016 3:11:13 PM

; Author : r

;

; REQUIREMENTS : Generate a 50% DC with 0.5sec period on PORTC.0

; FOR 8MHZ CLOCK

SBI DDRC, 0			; set PC0 output

CBI DDRC, 0			; clear PC0 bit

PC0_Blink:

CALL delay_250ms	; call 1/4 second delay procedure (current counter≈250ms)

LDI  R17, 1			; R17 = 1

OUT PORTC, R22		; set PC0 high

CALL delay_250ms	; call 1/4 second delay procedure (current counter≈500ms)

LDI  R17, 0			; R17 = 0

OUT PORTC, R22		; set PC0 low

JMP PC0_Blink		; repeat procedure

;	DELAY CALCULATION:

;	4 * 55 * 90 * 100 = 1,980,000

;	We are using 8MHz frequency, so 1,980,000 / 8,000,000 = 0.2475 ≈ 250ms

;

delay_250ms:

PUSH R18			; push R18

PUSH R19			; push R19

PUSH R20			; push R20 (registers for creating looping branch cycles for delay)

LDI R18, 55			; R18 = 55

delay1:

LDI R19, 90			; R19 = 90

delay2:

LDI R20, 100		; R20 = 100

delay3:

NOP					; NOP adds extra cycle

DEC R20

BRNE delay3			; repeats previous loops to add to clock cycle

DEC R19

BRNE delay2

DEC R18

BRNE delay1

POP R20				; pop back registers used for loops

POP R19

POP R18

RET					; finished delay procedure, go back to main

| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 1/B |  |  |
| --- | --- | --- | --- |

;

; DA2T2.asm

;

; Created: 3/8/2016 4:32:04 PM

; Author : User

;

; REQUIREMENTS:		Use PORTB to display an 8-bit binary counter. PC4 will toggle every 5th pulse

;					and PC5 will toggle every 10th pulse. Non-interrupt method.

SBI DDRB, 0			; output PB0

SBI DDRB, 1			; output PB1

SBI DDRB, 2			; output PB2

SBI DDRB, 3			; output PB3

SBI DDRB, 4			; output PB4

SBI DDRB, 5			; output PB5

SBI DDRB, 6			; output PB6

SBI DDRB, 7			; output PB7

SBI DDRC, 0			; output PC0 (pulse)

SBI DDRC, 4			; output PC4 (toggle 5th pulse)

SBI DDRC, 5			; output PC6 (toggle 10th pulse)

LDI R22, 0x00			; R22 is counter used for 8-bit counter

LDI R23, 0x00			; R23 is counter for 5th or 10th rising pulse check

binaryCounter:

CALL delay_250ms		; 250ms delay procedure

INC R22					; increment 8-bit counter

OUT PORTB, R22			; output 8-bit counter on PORTB

INC R23					; increment count for detecting 5th or 10th rising pulse

; PULSE CHECK PROCEDURE

CPI R23, 5			; first 5th pulse

BREQ rising_5		; branch if equal to 5

CPI R23, 10			; next 5th (10th pulse)

BREQ rising_10		; branch if equal to 10

CPI R23, 15			; next 5th (15th pulse)

BREQ rising_15		; branch if equal to 15

CPI R23, 20			; next 5th (20th pulse)

BREQ rising_20		; branch if equal to 15

JMP next_250ms			; if does not match any case, toggle nothing

rising_5:

LDI R24, 0x10

OUT PORTC, R24		; 5th pulse where PC4 = 1 and PC5 = 0

JMP next_250ms		; finish pulse output, complete the delay

rising_10:

LDI R24, 0x20

OUT PORTC, R24		; 10th pulse where PC4 = 0 and PC5 = 1

JMP next_250ms		; finish pulse output, complete the delay

rising_15:

LDI R24, 0x30

OUT PORTC, R24		; 5th pulse where PC4 = 1 and PC5 = 1

JMP next_250ms		; finish pulse output, complete the delay

rising_20:

LDI R24, 0x00

OUT PORTC, R24		; second 10th pulse where PC4 = 0 and PC5 = 0

LDI R23, 0x00		; set pulse check counter back to zero to redo cases

JMP next_250ms		; finish pulse output, complete the delay

next_250ms:

CALL delay_250ms	; delay another 250ms to complete 0.5sec period

JMP binaryCounter	; count again, don't care about overflow

;	DELAY CALCULATION:

;	4 * 55 * 90 * 100 = 1,980,000

;	We are using 8MHz frequency, so 1,980,000 / 8,000,000 = 0.2475 ≈ 250ms

;

delay_250ms:

PUSH R18			; push R18

PUSH R19			; push R19

PUSH R20			; push R20 (registers for creating looping branch cycles for delay)

LDI R18, 55			; R18 = 55

delay1:

LDI R19, 90			; R19 = 90

delay2:

LDI R20, 100		; R20 = 100

delay3:

NOP					; NOP adds extra cycle

DEC R20

BRNE delay3			; repeats previous loops to add to clock cycle

DEC R19

BRNE delay2

DEC R18

BRNE delay1

POP R20				; pop back registers used for loops

POP R19

POP R18

RET					; finished delay procedure, go back to main

| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 1/C |  |  |
| --- | --- | --- | --- |

;

; DA2T3.asm

;

; Created: 3/8/2016 4:59:29 PM

; Author : r

;

.ORG 0x00			; PC = 0x00

RJMP INIT			; Initialize Stack at PC = 0

.ORG 0x16			; TIM1_COMPA in interrupt table is 0x16

RJMP TIM1_COMPA		; Timer 1 Compare A Handler

INIT:				; Macro that initializes stack pointer

.MACRO INITSTACK

LDI	R16, HIGH(RAMEND)

OUT SPH, R16

LDI R16, LOW(RAMEND)

OUT SPL, R16

.ENDMACRO

INITSTACK

RJMP main

main:

SBI DDRB, 0			; output PB0

SBI DDRB, 1			; output PB1

SBI DDRB, 2			; output PB2

SBI DDRB, 3			; output PB3

SBI DDRB, 4			; output PB4

SBI DDRB, 5			; output PB5

SBI DDRB, 6			; output PB6

SBI DDRB, 7			; output PB7

SBI DDRC, 0			; output PC0 (pulse)

SBI DDRC, 4			; output PC4 (toggle 5th pulse)

SBI DDRC, 5			; output PC6 (toggle 10th pulse)

LDI R20, 0x00			; R20 is counter used for 8-bit counter

LDI R23, 0x00			; R23 is counter for 5th or 10th rising pulse check

LDI R24, 0x0F		; R24 used as temporary value register. load 0x0F into R24

STS OCR1AH, R24		; store 0x0F into the high register of OCR1A

LDI R24, 0x42		; load 0x42 into R24

STS OCR1AL, R24		; store 0x42 into the low register of OCR1A

LDI R24, 0x00		; load 0x00 into R24

STS TCCR1A, R24		; set CTC and have OC1A be disconnected to not change bits on bargraph

LDI R24, 0x0D		; load 0x0D into R24

STS TCCR1B, R24		; set prescaler to 1024 and WGM12 bit high

LDI R24, 0x02		; load 0x02 into R24

STS TIMSK1, R24		; set OCIEA1 high to enable interrupts for OCF1A

SEI					; enable global interrupts

loop:

RJMP loop	; relative jump, looping forever while interrupt occurs at 0.5 sec intervals

; INTERRUPT PROCEDURE

TIM1_COMPA:				; Timer 1 Compare A Handler

INC R20				; binary counter is incremented

OUT PORTB, R20		; update value of counter on LED bargraph

INC R23				; increment register checking for 5th/10th rising edge

; DETECT RISING PULSES

CPI R23, 5			; compare immediate, 5th pulse

BREQ rising_5		; if it is the 5th pulse, branch

CPI R23, 10			; compare immediate, 10th pulse

BREQ rising_10		; if it is the 10th pulse, branch

CPI R23, 15			; compare immediate, 15th pulse

BREQ rising_15		; if it is the 15th pulse, branch

CPI R23, 20			; compare immediate, 20th pulse

BREQ rising_20		; if it is the 20th pulse, branch

JMP negEdge			; jump to negative edge if not a 5th or 10th pulse

rising_5:

LDI R16, 0x10

OUT PORTC, R16	; PC4 = 0 and PC5 = 1, Toggle PC4

JMP negEdge		; toggle done, jump to negative edge

rising_10:

LDI R16, 0x20

OUT PORTC, R16	; PC4 = 1 and PC5 = 0, Toggle PC5

JMP negEdge		; toggle done, jump to negative edge

rising_15:

LDI R16, 0x30

OUT PORTC, R16	; PC4 = 1 and PC5 = 1, Toggle PC4 and PC5

JMP negEdge		; toggle done, jump to negative edge

rising_20:

LDI R16, 0x00

OUT PORTC, R16		; second 10th pulse where PC4 = 0 and PC5 = 0

LDI R23, 0x00		; set counter back to zero to redo toggle check

JMP negEdge		    ; toggle done, jump to negative edge

negEdge:

LDI R24, 0x02		; load 0x02 into R24

STS TIFR1, R24		; clear OCF1A flag

RETI				; return and enable interrupts

| 4. | INITIAL CODE OF TASK 2/A |  |  |
| --- | --- | --- | --- |

/*

* DA2T4.c

*

* Created: 3/8/2016 5:24:22 PM

* Author : r

*/

// REQUIREMENTS:	Blink PC0 with 50% DC and 0.5sec period.

#define F_CPU 8000000UL		// 8MHz Frequency for Simulator

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRC |= (1 << PC0);		// PC0 is output

while(1)				// loop forever blinking PC0

{

_delay_ms(250);		// 250ms delay

PORTC |= 0x01;		// set PC1 high

_delay_ms(250);		// 250ms delay (completes 0.5sec period)

PORTC = 0;			// set PC0 low

}

}

| 5. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| --- | --- | --- | --- |

/*

/*

* DA2T5.c

*

* Created: 3/8/2016 11:55:01 PM

* Author : r

*/

// REQUIREMENTS : Output 8-bit counter on PB0-PB7. 0.5sec delay. PC4 toggled every 5th

//				  rising edge and PC5 is toggled every 10th rising edge.

#define F_CPU 8000000UL		// 8MHz Frequency for Simulator

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

int binaryCounter = 0;		// set counter for LED bargraph starting at 0

int risingCounter = 0;		// set counter for 5th/10th rising edge starting at 0

DDRB = 0xFF;			// PB0-PB7 set to outputs

DDRC = 0x30;			// PC4 and PC5 set to outputs

while(1)				//count sequence loop forever

{

_delay_ms(250);		// 250ms delay

binaryCounter++;			// increment counter after delay

PORTB = binaryCounter;		// output counter to LED

risingCounter++;			//used to detect every 5th and 10th rising edge

if(risingCounter%5 == 0) //PC4 toggles every 5 rising pulses

PORTC ^= (1<<PC4);

if(risingCounter%10 == 0) //PC5 toggles every 10 rising pulses

PORTC ^= (1<<PC5);

_delay_ms(250);			// 250ms delay (completes 0.5sec period)

}

}

| 6. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/C |  |  |
| --- | --- | --- | --- |

/*

* DA2T6.c

*

* Created: 3/9/2016 12:13:03 AM

* Author : r

*/

// REQUIREMENTS : Output 8-bit counter on PB0-PB7. PC0 generates 50% DC and 0.5sec period.

//				  PC4 toggles every 5th rising pulse, PC5 toggles every 10th rising pulse.

//				  Toggles are handled by interrupts

#define F_CPU 8000000UL //set clock frequency to 16MHz for compiler

#include <avr/io.h>

#include <avr/interrupt.h>		//use interrupt library

void delay_msec(int ms); //delay function procedure

//calculation using clock freq. (here we are using 8MHz)

int main(void)

{

DDRC |= 0x31;	// set PC4 an PC5 as outputs on PORTC

DDRB |= 0xFF;	//set every pin on PORTB as an output

//////////////

//TIMER 0

//////////////

DDRD &= ~(1<<PD4); //set PORTD.4 as input. pin PD4 uses TIMER0 and PC0 connects to this pin

//for the binary counter to properly count on the LEDs.

TCCR0A = 0; //timer 0 in normal mode

TCCR0B = 0x07; //external clock on TIMER0

TCNT0 = 0;	//initialize binary count sequence

//////////////

//INTERRUPTS

//////////////

PCICR |= (1<<PCIE1); //turn on pin change interrupts for PORTC

PCIFR |= (1<<PCIF1); //turn on pin change interrupt flag for PORTC

PCMSK1 |= (1<<PCINT8); //turn on PC0 (PCINT8) for interrupt usage (PCMSK1 is for PORTC)

sei(); //set interrupts

//////////////

//BINARY COUNTER

//////////////

while(1) //generate waveform of 50% DC with 0.5 sec period

{

PORTC &= ~(1<<PC0);	//PORTC.0 low

delay_msec(250); //delay for 250 ms

PORTC |= (1<<PC0);	//PORTC.0 high

PORTB = TCNT0+1;	//binary count sequence incremented by 1 each loop

delay_msec(250); //delay for 250 ms

}

}

//////////////

//DELAY PROCEDURE

//////////////

//////////////

void delay_msec(int ms)

{

double timerCalc = 65536 - (((double)ms)/1000)*((double)F_CPU/64);

//set timer calculation = 2^16 - delay in sec / (clock period/ prescaler 64)

TCNT1H = ((int)timerCalc) >> 0x08; //move upper byte of TCNT

TCNT1L = ((int)timerCalc) & 0x00FF; //move lower byte of TCNT

TCCR1A = 0; //Timer 1 Normal Mode

TCCR1B = 0x03; //Prescaler = 64 (we can easily change prescaler here for emulation)

while(!(TIFR1 & (1<<TOV1))) {	//loop until overflow

}

TCCR1B = 0; // stop timer 1

TIFR1 = 1; // clear overflow flag

return;

}

//////////////

//INTERRUPT SERVICE ROUTINE

//////////////

ISR (PCINT1_vect) //PC0 is in the PCINT1_vect

{

if(!(PORTC & (1<<PC0)) == 0)	//if PC0 is changed from low to high

{

static int risingCounter = 0; //used to detect every 5th and 10th rising edge

risingCounter++;

if(risingCounter%5 == 0) //PC4 toggles every 5 rising pulses

PORTC ^= (1<<PC4);

if(risingCounter%10 == 0) //PC5 toggles every 10 rising pulses

PORTC ^= (1<<PC5);

}

return;

}

| 7. | SCHEMATIC |  |  |
| --- | --- | --- | --- |

| 8. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

Assembly Version – Simulation

Half Period -> 249.38ms		->	Cycle Counter = 1995058

Full Period -> 498.76ms ≈ 0.5 sec period	->	Cycle Counter = 3990107 (≈ 50% Duty Cycle)

C Version – Simulation

Half Period -> 250.10ms		->	Cycle Counter = 2000821

Full Period -> 500.26ms ≈ 0.5 sec period	->	Cycle Counter = 4002102(≈ 50% Duty Cycle)

| 9. | SCREENSHOT OF EACH DEMO |  |  |
| --- | --- | --- | --- |

From top to bottom on LED bar respectively: PB0, PB1, PB2, PB3, PB4, PB5, PB6, PB7, PC4, PC5. The 8-bit counter is displayed from PB0-PB7 and it counts from 0 to 255 (with overflow). PC4 toggles for every 5th pulse, and PC5 is toggled for every 10th pulse. The green LED connects to PC0 and it is used to generate a half second pulse for the counter (with 50% DC).

9th bit is toggled for every 5th rising pulse

10th bit is toggled for every 10th rising pulse

9th bit and 10th bit can both be toggled

| 10. | VIDEO LINK OF DEMO |  |  |
| --- | --- | --- | --- |
|  | https://www.youtube.com/watch?v=5Q0N-Fxzzh0 |  |  |

| 11. | GITHUB LINK OF THE DA |  |  |
| --- | --- | --- | --- |
|  | https://github.com/magor1/embedded-design-VM.git |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

RALPH MAGO