RALPH MAGO

CPE301 – SPRING 2016

# Design Assignment 3

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | FLOWCHART OF THE CODE | Y |  |
| 1. | COMPONENTS LIST | Y |  |
| 2. | INITIAL CODE OF TASK 1/A | Y |  |
| 3. | SCHEMATICS | Y |  |
| 4. | SCREENSHOTS OF EACH TASK OUTPUT | Y |  |
| 5. | SCREENSHOT OF EACH DEMO | Y |  |
| 6. | VIDEO LINKS OF EACH DEMO | Y |  |
| 7. | GITHUB LINK OF THE DA | Y |  |

| 0. | FLOWCHART OF THE CODE |  |  |
| --- | --- | --- | --- |

| 1. | COMPONENTS LIST |  |  |
| --- | --- | --- | --- |

| Atmel Xplained Mini (328P) | 1 |
| --- | --- |
| Sparkfun FTDI Basic Board | 1 |
| TI LM34 Temperature Sensor | 1 |
| PC running Atmel Studio Serial Terminal | 1 |

| 2. | INITIAL CODE OF TASK 1/A |  |  |
| --- | --- | --- | --- |

/*

* DA3T1.c

*

* Created: 3/24/2016 5:01:14 PM

*  Author: magor

*/

#define F_CPU 16000000UL //16 MHz clock

#define BAUD 9600		//9600 Baud Rate (default)

#define MYUBRR (double)F_CPU/16/BAUD - 1	//given baud rate calculation

#define TCNT1_VALUE 65536 - ((double)F_CPU/256) //calculation for Timer 1 Counter (1s Period)

#include <avr/io.h>

#include <avr/interrupt.h>

void init_Timer1 (void)

{

TCNT1 = TCNT1_VALUE; //overflow in 1 sec

TCCR1A = 0; //normal mode

TCCR1B = 4; //prescaler = 256

}

int main(void)

{

DDRC &= (0<<PORTC0);	//PC0 = ADC0 for ADC input

UCSR0B = (1<<TXEN0); //enable TX

UCSR0C = ((1<<UCSZ01)|(1<<UCSZ00)); //set Asynchronous Mode

UBRR0L = MYUBRR;	// Baud Rate 9600, 16MHZ

ADCSRA = 0x87;

//0b10000111, 7th bit set for ADEN. Bit 0-3 set to 111 for prescaler 128

ADMUX |= (1<<REFS0) | (1<<REFS1); //use internal VREF for LM34

sei(); //enable interrupt

init_Timer1 ();	//configure timer 1 to interrupt every second

TIMSK1	|= (1<<TOIE1); //enable Timer Overflow Interrupt

while(1);

{

// loop forever with interrupts

}

return 0;

}

void USART_TX_Out(char *ch) //takes a string and sends it serially

{

while(*ch != '\0')	//send chars until NULL is found

{

while(!(UCSR0A & (1<<UDRE0))); //wait for UDRE0 to be 1

UDR0 = *ch;	//send char serially

ch++; //point to next char

}

}

ISR(TIMER1_OVF_vect) //timer1 overflow ISR

{

TCCR1B = 0; //stop timer 1

TIFR1 = 1; //clear overflow flag

int adc_temp;	//stores ADC temporarily

float adc_temp_Float; //float for calculations

int adc_LeftSide; //left side of the value before decimal point

int adc_RightSide; //right side of the value before decimal point

ADCSRA |= (1<<ADSC); //ADC start conversion

while((ADCSRA &(1<<ADIF)) == 0); //while conversion isn't finished

adc_temp = ADC;	//get ADC value (LM34 Vout)

adc_temp_Float = (float)adc_temp * (1.1 / 1024) / 0.01;

//AREF = 1.1V, 10 bit ADC = 1024, 10mV°F Scale Factor

adc_LeftSide = (int)adc_temp_Float; //integer part before decimal point

adc_temp_Float = adc_temp_Float - adc_LeftSide; //subtract left side

adc_RightSide = (int)(adc_temp_Float * 10); //decimal part before decimal point

char string_temp[18] = "LM34 TEMPERATURE: ";

char TempInteger[27]; //stores temperature to be displayed

sprintf(TempInteger, "%d", adc_LeftSide); //integer pAart to string

strcat(TempInteger, "."); //concatenate dot

char TempDecimal[6]; //stores decimal part

sprintf(TempDecimal, "%d°F\r\n", adc_RightSide);

// %d displays integer value of decimal value

//output degrees Fahrenheit to the end of the

//transmission. \r\n combination of carriage return

//and line feed prevents scattered data in serial terminal

strcat(TempInteger, TempDecimal); //concatenate integer and decimal parts

USART_TX_Out(string_temp);	//display finalized temperature value string on serial terminal

init_Timer1 ();	//configure timer 1 to interrupt every second

return;

}

| 3. | SCHEMATIC |  |  |
| --- | --- | --- | --- |

| 4. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

Using Atmel Studio 7, I used the Terminal extension in order to access the serial terminal using a communication port. The FTDI basic was connected to my PC via USB 2.0.

Initial Start: Terminal Window is clear

Serial Port Connected: FTDI transmitting LM34 Temperature converted with ADC.

| 5. | SCREENSHOT OF EACH DEMO |  |  |
| --- | --- | --- | --- |

Breadboard, Atmel Xplained Mini 328P, and FTDI Basic connections:

| 6. | VIDEO LINK OF DEMO |  |  |
| --- | --- | --- | --- |
|  | https://www.youtube.com/watch?v=sbD2Pp3_FiA |  |  |

| 7. | GITHUB LINK OF THE DA |  |  |
| --- | --- | --- | --- |
|  | https://github.com/magor1/embedded-design-VM/tree/master/DA3 |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

RALPH MAGO