RALPH MAGO

CPE301 – SPRING 2016

# Design Assignment FINAL

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | GOAL OF THE PROJECT | Y |  |
| 1. | DELIVERABLES | Y |  |
| 2. | LITERATURE SURVEY | Y |  |
| 3. | COMPONENTS | Y |  |
| 4. | SCHEMATICS | Y |  |
| 5. | INITIAL PCB | Y |  |
| 6. | IMPLEMENTATION | Y |  |
| 7. | SCREENSHOTS AND VIDEO | Y |  |
| 8. | CODE | Y |  |
| 9. | REFERENCES | Y |  |

| 0. | GOAL OF THE PROJECT |  |  |
| --- | --- | --- | --- |

- Capture gyro/accelerometer data from muscle movements in the Z-axis, rotation around the X-axis, and rotation around the Y-axis.

- Display these movement patterns using a serial terminal with values transmitted wirelessly via Bluetooth.

- Because wireless implementation is possible, this device has an application in tracking muscle twitching patterns by storing data into a mobile device.

- Fully developed version of this circuit will be similar to a surface electrode which transmits muscle movements to a serial terminal wirelessly.

| 1. | DELIVERABLES |  |  |
| --- | --- | --- | --- |

This project is expected to deliver data from the gyro/accelerometer into a graphical output:

- Use Bluetooth Terminal/Graphics for Android for serial communication

  - Position: Z-axis gyro/accelerometer data (Graph 1)

  - Pitch : Measurement of rotation around X-axis (Graph 2)

  - Roll: Measurement of rotation around Y-axis (Graph 3)

- AVR MAIN C code + referenced .c and header files

- Schematics, PCB

- Demonstration of BT serial communication

| 2. | LITERATURE SURVEY |  |  |
| --- | --- | --- | --- |

Muscle movement detection is used for finding patterns in movement for animals (species research) and humans (muscle disease). In a research study [3], diaphragm contraction of alligators was measured in a graphical output. Muscle twitching patterns in humans [4] was observed in identifying muscle contraction and relaxation for normal human subjects and subjects with tetanus.

The current state of the project allows a functional graphical representation of gyro/accelerometer movement by the user, by wrist or ankle movement. Wireless transmission using Bluetooth to send the captured accelerometer data will be useful in observing movement data over time and finding rotational movement patterns along the X, Y, and Z axes.

| 3. | COMPONENTS |  |  |
| --- | --- | --- | --- |

| Atmel Xplained Mini (328P) | 1 |
| --- | --- |
| MPU6050 Gyro/Accelerometer | 1 |
| HC-06 Bluetooth Module | 1 |
| Power Supply (3.3V, 5V) | 1 |

| 4. | SCHEMATICS |  |  |
| --- | --- | --- | --- |

- ATMega328P has PC4 connected to SDA and PC5 connected to SCL on MPU-6050 (gyro/accelerator)

- ATMega328P has RX and TX connected with TX and RX with HC-06 (BT Module)

- ATMega328P and MPU-6050 connected to 5V power

- BT Module connected to 3.3V power

| 5. | INITIAL PCB |  |  |
| --- | --- | --- | --- |

- UI represents ATMega328P

- M1 represents HC-06 Bluetooth Module

- B1 represents MPU-6050 Gyro/Accelerometer

| 6. | IMPLEMENTATION |  |  |
| --- | --- | --- | --- |

- Prepare a Bluetooth capable device (Android) that can run a serial terminal to transmit and receive between the HC-06 Bluetooth Module and the microcontroller

- Make necessary conversions to gyro/accelerometer captured data in order for the serial terminal to get a correct output.

- Establish proper wireless connection through Bluetooth

- Enable graphs for Z-axis, Pitch, and Roll calculations

- Rotate accelerometer around and observe changes in output data.

- Interfaces:

  - ATMega328P

  - UART – TX and RX between HC-06 and Serial Terminal

    - Serial Terminal used is Bluetooth Terminal/Graphics on Android

  - I2C – HC-06 is the slave device

| 7. | SCREENSHOTS AND VIDEO |  |  |
| --- | --- | --- | --- |

- Position along Z-axis (red)

  - Graph spikes up when moving left

  - Graph spikes down when moving right

- Side-to-side rotation about X-axis aka Pitch (green)

  - Turning around X-axis results in a vector on the Y-axis

  - Graph spikes up rolling right

  - Graph spikes down rolling left

- Up-and-down rotation about Y-axis aka Roll (blue)

  - Turning around Y-axis results in a vector on the X-axis

  - Graph spikes up pitching upwards

  - Graph spikes down pitching downwards

- Muscle Movement Detection Unit – YouTube

  - https://www.youtube.com/watch?v=6vSMingkX18

| 8. | CODE |  |  |
| --- | --- | --- | --- |

- The header files are refenced from [2]

  - I2c.h / i2.c -> define functions for i2c operation

  - Mpu6050_reg.h -> defines register addresses of MPU6050

  - Mpu6050.h / mpu6050.c -> define functions to operate mpu6050

  - Uart.h / uart.c -> define functions to operate UART interface for serial comm.

/*

* DA7.c

*

* Created: 5/4/2016 12:48:55 AM

* Author : r

*/

// Muscle Movement Detection Unit

//

// Captures data from Gyro/Accelerometer

// Implements UART and I2C using BT

#define F_CPU 16000000UL	//clock frequency (Fosc)

#define BAUD 9600		// Baud Rate 9600, matching

#define GYRO_SENSITIVITY 65.536		// referenced from [1]

#define M_PI 3.14159265359		//pi

#include <avr/io.h>		//AVR input/output

#include <util/setbaud.h> //requires that entry values already defined for F_CPU and BAUD

#include <math.h>

#include <inttypes.h> //necessary to use uint types from stdint.h, and printf functions

#include <util/delay.h>	//_delay_ms procedure for millisecond delay

#include <avr/interrupt.h> //interrupt enable for serial terminal

#include "i2c.h"

#include "mpu6050.h"

#include "mpu6050_reg.h"

#include "uart.h"

void timer_setup();			// referenced from [2]

void get_time(double* dt);

volatile double count;

const double unit_t = 8/16000000;

int main(void){		//variable declarations and procedures referenced from [2]

sei();

uart_init();

i2c_init();

DDRB |= _BV(5);

uint8_t ret;

int16_t accel_buff[3], gyro_buff[3];

double accelX, accelY, accelZ;

double gyroX, gyroY, gyroZ;

double dt;

float pitch;	//angle around x axis

float roll;		//angle around y axis

float pitch_accel, roll_accel;

float pitch_count = 0;

float roll_count = 0;

int count_previous_x = 0;

int count_previous_y = 0;

// initialize & test MPU5060 availability

ret = i2c_start(MPU6050_ADDRESS+I2C_WRITE);

if(~ret){

PORTB |= _BV(5);

_delay_ms(200);

PORTB &= ~(_BV(5));

}

mpu6050_init();

timer_setup();

mpu6050_read_accel_ALL(accel_buff);

while(1){

get_time(&dt);

mpu6050_read_accel_ALL(accel_buff);

mpu6050_read_gyro_ALL(gyro_buff);

// acceleration (m/s^2)

accelX = accel_buff[0]*9.8*2/32768;

accelY = accel_buff[1]*9.8*2/32768;

accelZ = accel_buff[2]*9.8*2/32768;

// gyro rate (degrees/s)

gyroX = gyro_buff[0]*(180/M_PI)*1000/32768;

gyroY = gyro_buff[1]*(180/M_PI)*1000/32768;

gyroZ = gyro_buff[2]*(180/M_PI)*1000/32768;

/////////////////////////////////////////

// Complementary Filter referenced from [1]

/////////////////////////////////////////

// Integrate the gyroscope data -> int(angularSpeed) = angle

pitch = pitch + ((float)gyro_buff[0] / GYRO_SENSITIVITY) * dt;

roll = roll - ((float)gyro_buff[1] / GYRO_SENSITIVITY) * dt;

// Compensate for drift with accelerometer data if !bull

// Sensitivity = -2 to 2 G at 16Bit -> 2G = 32768 && 0.5G = 8192

int approx_force_magnitude = abs(accel_buff[0]) + abs(accel_buff[1]) + abs(accel_buff[2]);

if(approx_force_magnitude > 8192 && approx_force_magnitude < 32768)

{

// Turning around the X axis results in a vector on the Y-axis

// Also known as "Pitch"

pitch_accel = atan2f((float)accel_buff[1], (float)accel_buff[2]) * 180 / M_PI; //in degrees

pitch = pitch * 0.98 + pitch_accel * 0.02;

if((pitch > 25 || pitch <-25) && count_previous_x == 0)

{

pitch_count += 1;

count_previous_x = 1;

}

else if (!(pitch > 25 || pitch <-25)) count_previous_x = 0;

// Turning around the Y axis results in a vector on the X-axis

// Also known as "Roll"

roll_accel = atan2f((float)accel_buff[0], (float)accel_buff[2]) * 180 / M_PI; //in degrees

roll = roll * 0.98 + roll_accel * 0.02;

if((roll > 25 || roll <-25) && count_previous_y == 0)

{

roll_count += 1;

count_previous_y = 1;

}

else if(!(roll > 25 || roll <-25)) count_previous_y = 0;

}

//App: Bluetooth Terminal/Graphics (Android)

//The string output to the serial terminal must be in this form:

//"Eval1,val2,val3\n"

//Where val1, val2, ... , are the value variables and are separated by commas.

_delay_ms(5);

uart_putstring("E");		//first character of serial terminal transmission

uart_putdouble(gyroZ);		//display gyro data of Z axis

uart_putstring(",");		//separator

uart_putdouble(pitch);		//display pitch angle

uart_putstring(",");		//separator

uart_putdouble(roll);		//display roll angle

uart_putstring(",");		//separator

uart_putdouble(gyroX);		//OPTIONAL display gyro data of X axis

uart_putstring(",");		//separator

uart_putdouble(gyroY);		//OPTIONAL display gyro data of Y axis

uart_putstring("\n");		//end of string for input values

uart_putstring("                                                                  X-axis turns: ");

uart_putdouble(pitch_count);	//number of pitches

uart_putstring("        Y-axis turns: ");

uart_putdouble(roll_count);		//number of rolls

uart_putstring("\n");

}

}//end of main

void timer_setup(){		//referenced from [2]

TCCR1A = 0x00;

TIMSK1 |= _BV(TOIE1);

TCCR1B |= _BV(CS11);

TCCR1B &= ~( _BV(CS12)  | _BV(CS10)); // prescaler=8

}

void get_time(double * dt){  //referenced from [2]

cli();

uint8_t l = TCNT1L;

uint8_t h = TCNT1H;

uint16_t step = h<<8 | l;

*dt = (double)step*5e-7 + count*0.032768;

count = 0;

sei();

}

// timer 1 overflow interrupt handler

SIGNAL(TIMER1_OVF_vect){	//referenced from [2]

count += 1;

//TCNT1H = 0x00;

//TCNT1L = 0x00;

}

| 9. | REFERENCES |  |  |
| --- | --- | --- | --- |

[1]P. Jan. (Apr 2013). Reading a IMU Without Kalman: The Complementary Filter (1st ed.)[Online]. Available: http://www.pieter-jan.com/node/11

[2]Y. Jiang. (Dec 2014). MPU6050, GitHub repository [Online].

Available: https://github.com/YifanJiangPolyU/MPU6050

[3]T. J. Uriona, C. G. Farmer. (2008). Recruitment of the diaphragmaticus, ischiopubis and other respiratory muscles to control pitch and roll in the American alligator (Alligator mississippiensis) (1st ed.)[Online]. Available: http://jeb.biologists.org/content/211/7/1141

[4]K. Beitas et al. (2001). HUMAN BIOLOGY: Electrophysiological experiment (1st ed.)[Online]. Available: http://www.pef.uni-lj.si/eprolab/comlab/sttop/sttop-bm/bm-elephy.htm#Index

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

RALPH MAGO